Network User Guide Training Booklet

Chance Robson; 12/2/25; JustIT.

Contents

Adding Devices to the Network	2

Adding a Device with a Static IP Address	2

Adding a Device Using Dynamic IP (DHCP)	3

Connecting a Device Using a Cable	4

Connecting a Device Wirelessly	5

IoT Devices	6

Adding IoT Devices	6

Using the IoT Management App	7

Common IoT App Features	7

# Adding Devices to the Network

## Adding a Device with a Static IP Address

A static IP address is manually typed in and never changes.
Used for important devices such as:

- Printers

- IoT Server

- Critical PCs

Steps:

- Turn on the device and open the Network Settings menu.

- Select Manual / Static IP configuration.

- Enter the details provided by your IT team, for example:

  - IP Address: 192.168.25.xxx

  - Subnet Mask: 255.255.255.0

  - Gateway: 192.168.25.254

  - DNS Server: 192.168.25.1

- Click Save or Apply.

- Restart the device if required.

How do you know it worked?
It should appear like this.

Tip:

- To test the connection, Try to open a webpage. For example https://www.justit.co.uk

If that doesn’t work, Contact IT Support

## Adding a Device Using Dynamic IP (DHCP)

A dynamic IP address is automatically given by the wireless router.

Suitable for:

- Laptops

- Smartphones

- Most IoT devices

- Hot-desk computers

Steps:

- Open Network Settings.

- Choose Automatic / DHCP.

- Connect the device to the network (wired or wireless).

- It will automatically get an IP address from the router.

How do you know it worked?
It should appear like this

Tip:

- To test the connection, Try to open a webpage. For example https://www.justit.co.uk

If that doesn’t work, Contact IT Support

## Connecting a Device Using a Cable

What you need:

- A Ethernet cable provided by IT

- An available port

Steps:

- Plug one end of the cable into your computer or printer.

- Plug the other end into a network switch port.

- The port light will turn green or amber to show activity.

- You will be made aware if you require a static IP or DHCP. Depending on this, follow the instructions previously in the guide

Tip:

- If no lights appear, try a different port or cable.

If that doesn’t work, Contact IT Support

## Connecting a Device Wirelessly

What you need:

- Wi-Fi name- SmartOffice

- Password- Provided by IT

Steps:

- Open Wi-Fi settings on your laptop/phone.

- Select SmartOffice from the list of networks.

- Enter the password

- Wait for the device to show "Connected".

How do you know it worked?
It should appear like this.

Tip:

- If the password fails, check for capital letters.

- If Wi-Fi signal is weak, move closer to the router where possible.

- If the device asks for security type, choose WPA2.

If that doesn’t work, Contact IT Support

# IoT Devices

## Adding IoT Devices

Steps:

- Power on the IoT device.

- Open its Network Settings.

- Select SmartOffice Wi-Fi.

- Enter the password

- Leave IP settings on Automatic (DHCP).

- In the IoT Server field, enter:
192.168.25.1

- Save settings.

- Wait until the IoT Server shows the device as Online.

How do you know it worked?

It should appear like this

Tip:

- Ensure the IoT Server is powered on.

- Make sure Wi-Fi signal reaches the IoT device.

- Restart the device if it does not appear in the IoT app.

If that doesn’t work, Contact IT Support

## Using the IoT Management App

- Open the IoT Monitor App on your workphone.

- Make sure you are connected to SmartOffice Wi-Fi.

- Log in (if login is required).

- The app will show a list of all IoT devices:

  - Coffee machine

  - Fan

  - Light

  - Motion sensor

- Select a device to perform actions

Common IoT App Features

| Feature | What it does |
| --- | --- |
| Device Status | Shows if a device is online or offline |
| Control Button | Turns the device on/off |
| Sensor Data | Shows temperature, motion, or other readings |
| Automation | Schedules or triggers behaviours |

How do you know it worked?

It should appear like this

If that doesn’t work, Contact IT Support